Using GPIB plotters from Microsoft Windows

GPIB plotters can be used as the graphics output (print) device

with MS-Windows.  Simply follow these steps:

1) Re-direct output to the HPIB plotter with the

PRINTER program.  For example, to redirect LPT2

output to the plotter, use:

PRINTER P1 5

PRINTER is provided on the PC<>488 disk.

The Utilities appendix in the manual describes this program

in more detail.

2) Edit the windows configuration file "WIN.INI".

Find the line in this file which reads "[ports]".

Following this line you will see a list of port names,

such as COM1, COM2, etc.

Add a new line to this list:

If you redirect:          Add this line:

LPT1                   LPT1.PRN=

LPT2                   LPT2.PRN=

LPT3                   LPT3.PRN=

3) Run windows and select the control panel application.

Select "connections" from the menu.  Connect your printer

device to the new port choice you added in step 2.

Configure this device as the appropriate plotter model

(HP7470A, etc.)

That's it!