Using CEC-488 boards in Microsoft Windows

------------------------

For writing 16-bit Windows programs:

-------------------------

1)   Copy the file WIN488.DLL from the \WINDOWS directory

on the CEC-488 software disk to the Windows directory

on your computer.

For C or C++ programming:

Write your code as described in the manual.

Link with the library file:

WIN488.LIB

instead of the DOS library (IEEE488.LIB).

For Visual BASIC programming:

a) Copy the file IEEEVB.BAS from the \VB directory

on the CEC-488 software disk to any directory of your

choice on your computer.

b) Use the File/Add menu in VB to add this file to

your Visual BASIC project.

c) Write your code according to the manual, just as

for QuickBASIC, except that Enter and Receive are

slightly different.  See Appendix K for more details.

NOTE: For Visual BASIC 3.0 and earlier, use the file

IEEEVB3.BAS instead of IEEEVB.BAS!

For writing 32-bit Windows programs:

-------------------------

1)   Copy the file IEEE_32M.DLL from the \WIN32 directory

on the CEC-488 software disk to the Windows directory

on your computer.

2)   If you are using Windows NT (not Windows 95), run the

SETUPNT.EXE program from the \WIN32 directory of the

CEC-488 software disk.  This program will install

the Windows NT driver.

For C or C++ programming:

Write your code as described in the manual.

However, use this include file instead of ieee-c.h:

#include "ieee_32.h"

This include file may be found in the \WIN32 directory

of the CEC-488 disk.

Link with one of these libraries (depending on your

compiler), instead of with the DOS library (IEEE488.LIB):

IEEE_32M.LIB            for Microsoft Visual C++

or

IEEE_32B.LIB            for Borland C++

These libraries are found in the \WIN32 directory of

the CEC-488 disk.

For Visual BASIC programming:

See the instructions above.

No changes are needed for 32-bit programming.