HPIB plotters and AutoCAD

AutoCAD for Windows:

See the file PLOTTER.DOC for information on configuring

HPIB plotters in Windows for use with any programs.

Instructions for using HPIB (IEEE-488, GPIB) plotters with

AutoCAD(r) release 9 or later, in DOS:

1) Install the AutoCAD software on your computer according to the

usual instructions in the AutoCAD manual.

2) Install your PC-488 interface card according to the

instructions in the PC-488 manual.

3) Copy the file PRINTER.EXE from the disk supplied with your PC-

488 interface card to your AutoCAD working disk (normally your

hard disk).  The file should be placed in the root directory

(see your DOS manual for an explanation of directories).

For example:

copy a:\utility\printer.exe c:\dos

4) Determine the HPIB address of your plotter.  For Hewlett-Packard

plotters, the address is usually 5 (five). The address is

determined by switches on the plotter. The rest of these

instructions assume you have a parallel printer port on your

computer (PRN or LPT1). The PRINTER software will make AutoCAD

think that the HPIB plotter is on LPT2.

5) Add a line to your AUTOEXEC.BAT file on your disk to run the

PRINTER program:

PRINTER P1 5

If you don't already have a file named \AUTOEXEC.BAT, you should

create one.  You can create or modify this file with any word

processor software you have, even the EDLIN program supplied

with DOS.  If your word processor software has a "non-document"

mode for storing files, use it. In the sample command line

given above, P1 refers to parallel port #1, which will remain

as LPT1.  The numeral 5 refers to the HPIB address of your

plotter, which will become LPT2. If your plotter is at a

different address, use that number instead.

6) Re-boot your PC to install the PRINTER software.  This

installation will occur automatically each time you turn your

PC on.  Notice that PRINTER reports the configuration of the

ports when it runs.

7) Run AutoCAD, according to the usual instructions.

8) Choose "Configure Autocad" from the main menu. Next, choose

"Configure plotter".  Answer the questions just as you normally

would, until you get to the prompt "Write the plot to a file

<N>: ".  Type "Y" and hit return.  This will cause AutoCAD to

ask for a file name whenever you try to plot a drawing.

Finally, exit the configuration menu and save the new

configuration.

9) Now you're ready to go!  Just run AutoCAD the way you normally

would. When you want to plot a drawing, AutoCAD will ask you

for the file name to plot to.  Type "LPT2" and return, and the

plotter will go.

* AutoCAD is a trademark of Autodesk, Inc.