THE CECDRIVE.EXE, GPIB.DLL, and GPIB-32.DLL UTILITIES

------------------------------------------------

The CECDRIVE.EXE program is designed to enhance the

PC<>488, 488EX, 4x488, and PS<>488

products from Capital Equipment Corp.

by allowing application programs written for

the NI GPIB cards to run without modification.

This allows our boards to run any combination of programs:

those written specifically for CEC cards as well as those

written for NI cards, including LabWindows and LabView.

FILES INCLUDED

--------------------------------------------------

CECDRIVE.EXE            DOS support for NI compatibility

GPIB.DLL                MS Windows support for NI compatibility

GPIB-32.DLL             MS 32-bit Windows support for NI compatibility

GPIB.INI                Configuration settings file

CECDRIVE.DOC            This documentation file (text).

DOS INSTALLATION

--------------------------------------------------

Files are normally installed as part of standard CEC488 software

installation, using SETUP.

Typically, files are places in the C:\CEC488 directory.

1) Add the following command line to your AUTOEXEC.BAT file:

C:\CEC488\CECDRIVE\CECDRIVE

That's it!  Once CECDRIVE runs, it is installed in memory.

REMOVING CECDRIVE FROM MEMORY

------------------------------------------------------------

If you no longer need CECDRIVE, and want to free up the

memory it is using, use the command:

C:\CEC488\CECDRIVE>  CECDRIVE /R

WINDOWS INSTALLATION

------------------------------------------------------------

Files are automatically installed when you run SETUP from

the CEC488 software disk.

The required .DLL files are placed in your Windows directory.

RE-CONFIGURING CECDRIVE

----------------------------------------------------

For some application programs, the default configuration

of CECDRIVE will work.

However, some programs require that the driver be configured

with non-default device names or device settings.

To modify the CECDRIVE configuration, edit the file

GPIB.INI.

For DOS, CECDRIVE.EXE will find GPIB.INI by searching the DOS

PATH (this may include looking in the Windows directory, if

you have Windows installed.)

After you have modified GPIB.INI, run CECDRIVE again so it

will read the file again and modify its settings.

For Windows, GPIB.DLL will find GPIB.INI in the Windows

directory.

To avoid confusion, keep only one copy of GPIB.INI on your

disk.  If you have multiple copies, and modify the copy that

is not being used by CECDRIVE, the modifications will have

no effect.

GPIB.INI is an ASCII text file containing the settings for

one or more GPIB boards and up to 32 GPIB devices.

Each device section begins with a label, like this:

[DEV1]

To change the name of an existing device entry, just edit

this label.  Leave the square brackets - they are not part

of the device name.

In each section, there are settings, given as a setting name

followed by an equals sign, followed by the value.

The file contains comments describing the allowed choices

for values.

MULTIPLE CECDRIVE CONFIGURATIONS (CONFIGURATION FILES)

---------------------------------------------------

CECDRIVE.EXE can load a configuration file of your choice

instead of the default file GPIB.INI.

Just specify the file name on the CECDRIVE command line.

You can modify the configuration of CECDRIVE even if it

is already loaded, just by giving the CECDRIVE command

again, with a new configuration file name.

For example:

C>  CECDRIVE mygpib.cfg

NOTE:

Do not name the configuration file with the same name as

one of your device names in the file.

(For example, don't set a device name to [SCOPE] and then

call the file SCOPE.CFG).

This would cause problems, because CECDRIVE adds the device

names to DOS' list of devices, and DOS does not allow

devices and disk files to have the same name (regardless

of the .xxx extension).

USING AN EXISTING NI CONFIGURATION FILE

--------------------------------------------------------

If you have a running system under Windows using an NI

board, you can copy its GPIB.INI file for use with

our GPIB.DLL.  The formats are compatible.

USING AN OLD FORMAT CECDRIVE CONFIGURATION FILE

--------------------------------------------------------

Previous versions of CECDRIVE used a different configuration

file format and command line option. These will still work

with the current version of CECDRIVE.

For example:

CECDRIVE  /CONFIG=lecroy.cfg