------------------------------------------------------

Using CEC 488 interfaces with other operating systems

------------------------------------------------------

(This discussion is fairly technical, because it is designed to cover

any possible operating system, even one custom-written for your

application.  If you have specific questions about an operating

system, gather any technical data you can on the requirements listed

below and give us a call at CEC and we'll help you.)

CEC 488 software support is provided for the DOS, Windows,

Windows 95, Windows NT, and OS/2 operating systems.

Supports for some varieties of UNIX are also available separately.

Many programming languages are supported in these environments.

The software that comes with your board can be used with other

operating systems, as long as certain conditions are met:

1) You must be able to use the Microsoft library file

IEEE488.LIB in building your application programs.

If your operating environment allows you to write code using the

standard Microsoft C compiler and linker, or provides equivalent

tools that use the same file formats, you can use the library file

provided with CEC 488.

2) Your program must have the ability (permission) to execute

input/output instructions.

Any program can do I/O operations in DOS, or in any system which

runs the computer's processor in "real" mode. If your system runs

on an 80286 or 80386 in "protected" mode, some restrictions may

apply.  In order to use CEC 488, your operating system must allow

an application program (or at least the IEEE 488 code segment) to

do direct I/O operations.  In OS/2, for example, I/O privilege can

be granted to specific code segments when the program is linked.

3) The library does make one BIOS function call (software

interrupt 15 hex, function C0 hex), which must be allowed.

If your operating system does not allow an application program to

make this call, you can set the variable ieee488_os2mode to 1 in an

initialization procedure (see below).  Note that setting

ieee488_os2mode to 1 disables the use of DMA, and requires you to

set ieee488_ps2 to 1 in your initialization procedure IF you are

running on a Microchannel (PS/2) computer.

4) The CEC 488 library needs read access to a system timer

memory location.

A single word memory location must be available which increments on

a periodic timebase.  The default location assumed in segment 40

hex, offset 6C hex, which is used by the normal PC BIOS for time

counting.  The default increment rate is once every 55 msec.

If these assumptions are not correct for your system, you can

change the value of the timer location and rate in an

initialization procedure (see below). If your system runs in

"protected" mode, make sure that the program has read access to the

segment selector used in the timer location pointer.

Initialization procedures:

--------------------------

The CEC 488 library consists of two segments, a code

segment named ieee488_text, and a data segment named

ieee488_data, declared as shown below:

ieee488_text segment public 'CODE'

ieee488_data segment public 'DATA'

Certain variables in the data segment are preset to

match the normal DOS environment.

IF you need to change these values for your environment,

you can create an initialization procedure which is

called BEFORE you call any of the routines in the CEC

488 library.

The variables you may wish to change are shown below:

ieee488_timerdata dd 40006CH  ; pointer to timer word

ieee488_timer     dw 55       ; msec/timer tick

ieee488_ps2       db 0        ; set to 1 for Microchannel

; (PS/2) computer

ieee488_os2mode   db 0        ; set to 1 for OS/2

; (this only disables DMA,

; and skips setting ieee488_ps2

; by making an INT 15H call

; to the PC BIOS)

ieee488_segincr   dw 1000H    ; amount to increment

; segment selector by when

; offset wraps around

; at 64K (see #5, below)

An example of an initialization procedure is shown:

ieee488_data segment public 'DATA'

extrn   ieee488_timer:word

ieee488_data ends

_TEXT segment public 'CODE'

assume cs:_TEXT

public _init_488lib

_init_488lib proc near

mov     ax,ieee488_data         ; set segment

mov     es,ax

assume  es:ieee488_data

mov     ieee488_timer,30        ; set timer rate

;  to 30 msec/tick

ret                             ; return

_init_488lib endp

_TEXT ends

end

This would be called in C as:

init_488lib();

Additional notes for "protected" mode:

--------------------------------------

5) The TARRAY and RARRAY procedures allow "huge" pointers

to variables which may span more than 64K bytes.  In

order to handle this, these procedures increment the

segment portion of the memory address whenever the

offset portion overflows.  In "real" mode, the correct

increment value is always 64K bytes, or 1000H paragraphs

of 16 bytes each, so the variable ieee488_segincr is

set the 1000H by default.

In protected mode, you have two choices.  One is to avoid

using "huge" variables which cause the offset to wrap

around at 64K.

The other choice is to make sure the valid segment

selectors for the data area involved are in sequence,

with some constant increment.  Set the variable

ieee488_segincr to that increment value in an

initialization routine.  This is the

approach used in OS/2, which provides a system call to

obtain the correct increment value.

6) DMA (direct memory access) always uses absolute physical

addressing.  Therefore DMA must be used cautiously if

at all when operating in protected mode.

One choice is to avoid using DMA at all.  Just don't call

the dmachannel() procedure to enable it.

The other choice is to use DMA by enabling it and then

calling rarray() or tarray().  In this case, the data

pointers passed to these procedures MUST be "real" mode

memory addresses.  You will need a method of converting

proctected mode pointers to real mode addresses, and

making sure that the memory region is not swapped out

to disk or otherwise re-used during the DMA operation.