Calling 488 subroutines in ROM directly

---------------------------------------

PC<>488 and 488EX contain read-only-memories (ROM) which have

488 subroutines in them.

(The ROM in 488EX must be enabled by turning switch 1 position

7 ON.)

If you have a language or application which cannot use the

Microsoft library version of the 488 support routines (IEEE488.LIB),

you may be able to call the ROM routines directly.

The ROM routines are called by pushing the arguments on the

processor stack in left-to-right order.  All arguments are passed

as 16-bit offset values within the current data segment (DS

register).  This implies that all arguments MUST be within the

same 64K segment of memory.

String arguments are passed as a 16-bit offset of a string descriptor

in memory.  The descriptor itself consists of a 16-bit string

length followed by a 16-bit offset of the string data.

Below are examples of calls written in Microsoft assembler:

ROM     segment at 0CC00H

org     0

initialize proc far

initialize endp

org     36

send    proc    far

send    endp

ROM     ends

DATA    segment

myaddr  dw      21

level   dw      0

addr    dw      3

str     db      'MEASURE'       ; string

desc    dw      $-str           ; descriptor - length

dw      offset str      ;            - address

status  dw      ?

DATA    ends

CODE    segment

assume  cs:CODE,ds:DATA

mov     ax,DATA

mov     ds,ax

mov     ax,offset myaddr ; push offsets of args on stack

push    ax

mov     ax,offset level

push    ax

call    initialize      ; FAR call to ROM

mov     ax,offset addr

push    ax

mov     ax,offset desc

push    ax

mov     ax,offset status

push    ax

call    send

CODE    ends

end

Other routines are called in a similar manner.

The origin for each routine within the ROM segment is:

initialize      0

send            36

enter           39

transmit        30

receive         33

spoll           12

ppoll           15

tarray          200

rarray          203

dma2            206