Visual BASIC for DOS and CEC-488

----------

VBDOS may be used with CEC IEEE-488 cards.

All programming is identical to QuickBASIC, as described in the

CEC-488 manual.

Files required from the CEC-488 disks:

\VBDOS\IEEEVBD.QLB

\VBDOS\IEEEVBD.LIB

\VBDOS\IEEEVBD.BAS

\VBDOS\IEEEVBD.BI

To load the library when you start Visual BASIC:

VBDOS /L IEEEVBD.QLB

Make sure to put this line at the beginning of your program:

'$INCLUDE: 'IEEEVBD.BI'

If you need to rebuild the .QLB and .LIB files, use these commands:

BC IEEEVBD;

COPY IEEE488.LIB IEEEVBD.LIB

LIB IEEEVBD.LIB+IEEEVBD.OBJ;

LINK /Q IEEEVBD.LIB,IEEEVBD.QLB,,VBDOSQLB.LIB;

(The file IEEE488.LIB comes from the CEC-488 disk.

Files BC, LIB, LINK, and VBDOSQLB.LIB come from Visual BASIC).